CIVE 202

Spring 2026

Project 5

Written Report

North Dakota Pipeline Water Quality Analysis

The goal of the project was to evaluate water quality data for a given, proposed pipeline project in western North Dakota. The project considers water from Lake Sakakawea and the McClusky Canal, which would be pumped through a pipeline across the Sheyenne River and into Lake Ashtabula. Before the pipeline can be approved, the client requests to know if the water quality changes along the canal and if the mixed water entering Lake Ashtabula could create concerns.

This project was completed for the Garrison Diversion Project and the Red River Valley Water Supply Project. The main goal was to use water quality data to identify possible issues with major cations, major anions, total dissolved solids, pH, hardness, and other important parameters. These values are important because they describe the chemical condition of the water and help determine whether it may cause environmental, treatment, or long-term water quality problems.

Methods

The analysis was completed using Python. Required packages within Juypter such as pandas, numpy, matplotlib, seaborn, and plotly were utilized to properly and accurately clean the data, calculate summary statistics, and produce quality, readable graphs. The data was first loaded from the given USGS water quality file. Next, this code displayed each of the column names in a way that provided the important fields so they could be identified. These fields include respective location, water quality parameters, and measured result values.

The parameter regarding total dissolved solids (TDS), was one of these important fields that were considered in the analysis. TDS measures the amount of dissolved material in the given bodies of water, such as salt and mineral particles. A higher TDS value raises concern as these dissolved materials could affect water taste, treatment needs, irrigation use, and overall environmental issues. Our code filtered the data for TDS and grouped these results by body of water location to compare Lake Sakakawea, Lake Ashtabula, the Sheyenne River, and other, given canal sampling points.

Results

The analysis showed that water quality is variable dependent on location. Lake Sakakawea had an average TDS value of 394.60 mg/L, while other locations contained TDS values much above this number. Some select sampling locations had average TDS values above 700 mg/L, 900 mg/L, and even 1200 mg/L. These differing TDS values between the bodies of water show that dissolved solids are not consistent throughout the water system and that water quality is more affected by the body of water it is located in.

The created mass balance model gave an example mixing result where total flow into Lake Ashtabula was 13.00 cfs, in which the estimated TDS concentration after mixing was 530.77 mg/L. This result illustrates how the model can give somewhat accurate predictions regarding Lake Ashtabula water quality when foreign water flow and concentration values are given.

Graphs were created to help with data understanding, making our results easier to visualize and make sense of. Boxplots were made for TDS, sulfate concentration, sodium concentration, and chloride concentration. These generated boxplots provided the median value, general spread, and existing outliers for each body of water. TDS values differed noticeably between bodies of water, with some locations having larger ranges than others. Sulfate and sodium also showed major and consistent differences across the bodies of water system. Chloride values were generally lower, but some locations still had higher values or outliers.

Summary statistics were calculated for the main water quality parameters. Across the selected data, TDS had a mean of 809.31 mg/L, median of 777.00 mg/L, maximum of 1540.00 mg/L, and minimum of 323.00 mg/L. This given range, which is noticeably wide, shows that TDS should be one of the, if not the main parameter the client needs to continuously monitor. Other parameters, including bicarbonate, calcium, magnesium, sulfate, sodium, chloride, pH, hardness, and sodium adsorption ratio, were also summarized and can be deemed important to monitor throughout.

Discussion

Overall, the results show that water quality noticeably changed from Lake Sakakawea through the canal system. Many of the previously discussed parameters, especially TDS, sulfate, sodium, and chloride, differed by location without any noticeable pattern. These parameters need to be considered within this request as they are connected to salts and dissolved minerals in the water. If these values become severely high, they will raise great concern regarding water treatment, irrigation, environmental and marine life, and the overarching condition of Lake Ashtabula now, and within the future.

The mass balance model was created to allow the client experiment with different pipeline and river conditions, with general accuracy through created formulas and code. Since flow rates and concentrations will change over time due to things such as drought, season change, or water source variability, the model can give a general prediction for potential future outcomes, which allows the client to have many sources to look at, instead of one singular condition.

There are limitations to the analysis. Some bodies of water have more samples than others, bodies, possibly due to general size and popularity of the body of water, which could affect general data such as median and averages, as well as boxplots. Additionally, focusing on only the parameters discussed leaves out other potential water quality variables. To reduce this issue, the project included averages, medians, maximums, minimums, and boxplots so overall trends and general outliers could be pointed out.

Recommendations and Conclusion

The client should continue monitoring TDS, sulfate, sodium, chloride, hardness, and pH prior to approving and eventually operating the pipeline. These parameters proved obvious variability across multiple bodies of water and will affect the final receiving water in Lake Ashtabula. The client should utilize the mass balance model to consider differing flow rate and concentration scenarios. If predicted concentrations become extreme, the client is able to manually adjust flow rates, increase monitoring or come up with new techniques, or consider water treatment plans.

This project used Python to analyze water quality data for the proposed North Dakota pipeline. The results showed that water quality varies by body of water, especially for TDS and other salt/mineral adjacent parameters. Our graphical analysis, summary statistics, and mass balance model help explain and understand where concerns may lie and how this proposed pipeline could affect Lake Ashtabula. Based on our analysis, the pipeline could be possible, but continuous monitoring and further scenario testing should be completed before considering the final verdict.